NETWORKING &

THE OSI MODEL

A Complete Study Guide for Cyber Operations

Alex Rivera  |  Cyber Defense & Incident Response  |  2026

CONTENTS

Chapter 1  —  What is Networking?

Goals, PAN / LAN / MAN / WAN

Chapter 2  —  IP Addresses Deep Dive

Octets, Classes A–E, IPv4 vs IPv6, NAT & DHCP

Chapter 3  —  The OSI Model

All 7 Layers, Protocols, Data Flow

Chapter 4  —  Key Protocols & Port Numbers

TCP vs UDP, Essential Ports, DNS / DHCP / HTTP(S)

Chapter 5  —  Servers, Clients & Network Devices

Roles, Routers, Switches, Firewalls

Chapter 6  —  Security Concepts

CIA Triad, Common Attacks, Defence Tips

CHAPTER 1

What is Networking?

Networking is the practice of connecting devices so they can communicate and share resources. Without networking, every device would be an island — your laptop couldn't reach Google, your phone couldn't send a message, and your printer couldn't receive a document from across the room.

## Goals of Networking

- Share Resources — Files, printers, internet connections, all shared efficiently across devices.

- Enable Communication — Email, messaging, VoIP calls, all rely on network protocols.

- Remote Access — Access data stored on another machine, anywhere in the world.

- Centralised Management — IT teams can manage hundreds of devices from a single location.

## Types of Networks

Networks are categorised by the geographical area they cover. From your earphones to the entire planet, each type has a different scale and use case.

| Type | Full Name | Coverage | Example |
| --- | --- | --- | --- |
| PAN | Personal Area Network | ~1–10 metres | Bluetooth, AirDrop |
| LAN | Local Area Network | Building / ~100m | Home Wi-Fi, office network |
| MAN | Metropolitan Area Network | City / Town | City broadband, cable TV |
| WAN | Wide Area Network | Worldwide | The Internet |

💡 Memory Tip: PAN → LAN → MAN → WAN goes from smallest to largest — think of a zoom-out from your pocket to the whole globe.

CHAPTER 2

IP Addresses Deep Dive

An IP (Internet Protocol) address is a unique numerical label assigned to every device on a network. Think of it as a postal address — without it, data packets would have no idea where to go.

## Anatomy of an IP Address

An IPv4 address like 192.168.0.1 is made of four octets (8-bit numbers) separated by dots:

| Position | Value | Bits | Range |
| --- | --- | --- | --- |
| 1st Octet | 192 | 8 bits | 0 – 255 |
| 2nd Octet | 168 | 8 bits | 0 – 255 |
| 3rd Octet | 0 | 8 bits | 0 – 255 |
| 4th Octet | 1 | 8 bits | 0 – 255 |

Total: 8 x 4 = 32 bits  ->  2^32 = approximately 4.3 billion unique addresses.

## IP Address Classes

IPv4 addresses are divided into classes based on the value of the first octet. Each class serves a different scale of organisation:

| Class | Range | Network Bits | Max Hosts/Network | Typical Use |
| --- | --- | --- | --- | --- |
| A | 1 – 126 | 8 bits | ~16.7 million | Governments, large corps |
| B | 128 – 191 | 16 bits | ~65,536 | Universities, ISPs |
| C | 192 – 223 | 24 bits | 254 | Home / small office |
| D | 224 – 239 | N/A | N/A | Multicast / streaming |
| E | 240 – 255 | N/A | N/A | Reserved / research |

❓ Why is 127 skipped in Class A? 127.x.x.x is the loopback range. Pinging 127.0.0.1 (localhost) tests your own network stack without sending data onto the network. It never leaves your machine.

## IPv4 vs IPv6

IPv4's ~4.3 billion addresses seemed enormous in 1983, but with billions of IoT devices, it ran out. Enter IPv6:

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address Length | 32 bits | 128 bits |
| Total Addresses | ~4.3 billion | 340 undecillion (3.4 x 10^38) |
| Format | 192.168.0.1 | 2001:0db8:85a3::8a2e:0370:7334 |
| NAT Required? | Yes (mostly) | No |

🔑 What is NAT? Network Address Translation lets your router give private IPs (e.g. 192.168.x.x) to every home device while sharing a single public IP with the internet. This is why millions of households can each have a 192.168.0.1 router without conflict.

## DHCP — Automatic IP Assignment

DHCP (Dynamic Host Configuration Protocol) is the service inside your router that automatically hands out IP addresses. When your phone joins a Wi-Fi network, it broadcasts a request and DHCP replies with an available address — no manual configuration needed.

- Dynamic IP — Assigned automatically by DHCP. May change each session. Good for everyday devices.

- Static IP — Manually set. Never changes. Essential for servers, printers, and network cameras.

CHAPTER 3

The OSI Model

Open Systems Interconnection — the universal language of networking

The OSI model is a conceptual framework that describes how data travels from one device to another across a network. It breaks the journey into 7 distinct layers, each with a specific job.

When you hit Enter on a Google search, your data travels DOWN all 7 layers on your machine, across the network, then back UP the 7 layers on Google's servers.

## The 7 Layers

| # | Layer | Core Job | Key Protocols / Media |
| --- | --- | --- | --- |
| 7 | Application | Your apps live here | HTTP, HTTPS, FTP, SMTP, DNS |
| 6 | Presentation | Translates, encrypts, compresses | SSL/TLS, JPEG, ASCII, MPEG |
| 5 | Session | Opens and manages connections | NetBIOS, RPC, PPTP |
| 4 | Transport | Reliable delivery, segmentation | TCP, UDP |
| 3 | Network | Logical addressing and routing | IP, ICMP, OSPF, BGP |
| 2 | Data Link | Node-to-node transfer, MAC addr. | Ethernet, Wi-Fi (802.11), ARP |
| 1 | Physical | Electrical/optical signals | Cables, fibre optic, Wi-Fi radio |

Layer 7 Application — Defines the protocols apps use to communicate. When you open a browser and type a URL, the Application layer handles that request. Not the app itself, but the rules it uses to talk.

Layer 6 Presentation — Formats data so both sides understand it. Encryption (TLS/SSL) also happens here — this is what puts the 'S' in HTTPS.

Layer 5 Session — Manages sessions between applications — like keeping you logged into a website so you don't get signed out between clicks.

Layer 4 Transport — Breaks data into segments. TCP guarantees delivery and order (web, email). UDP is faster with no guarantee (video streaming, gaming).

Layer 3 Network — Assigns logical IP addresses and determines the best path across multiple networks. This is where your router operates.

Layer 2 Data Link — Handles communication between two directly connected devices using MAC addresses (hardware IDs). Switches operate at this layer.

Layer 1 Physical — The raw bits (0s and 1s) as electrical signals, light pulses, or radio waves. Defines cables, pins, voltages, and frequencies.

💡 Memory Mnemonic (Layer 7 to 1): All People Seem To Need Data Processing (Application · Presentation · Session · Transport · Network · Data Link · Physical)

CHAPTER 4

Key Protocols & Port Numbers

Protocols are agreed-upon rules that define how data is transmitted. Port numbers identify which specific service on a machine should receive the data.

## TCP vs UDP

Both operate at OSI Layer 4 (Transport) but have very different philosophies:

| Feature | TCP | UDP |
| --- | --- | --- |
| Full Name | Transmission Control Protocol | User Datagram Protocol |
| Connection | Connection-oriented (3-way handshake) | Connectionless |
| Reliability | Guaranteed delivery, ordered | No guarantee, no order |
| Speed | Slower (overhead) | Faster (low overhead) |
| Use Cases | Web (HTTP/S), email, SSH, FTP | Gaming, streaming, DNS, VoIP |
| Error Checking | Yes — retransmits lost packets | Minimal |

## Essential Port Numbers

Ports are virtual endpoints. Think of an IP address as a building and the port as the specific office number inside it.

| Port | Protocol | Service | Notes |
| --- | --- | --- | --- |
| 20/21 | TCP | FTP | File Transfer (21=control, 20=data) |
| 22 | TCP | SSH | Secure remote shell — always prefer over Telnet |
| 23 | TCP | Telnet | Insecure remote shell — avoid in production! |
| 25 | TCP | SMTP | Sending email |
| 53 | TCP/UDP | DNS | Domain name to IP resolution |
| 67/68 | UDP | DHCP | Automatic IP address assignment |
| 80 | TCP | HTTP | Unencrypted web traffic |
| 110 | TCP | POP3 | Receiving email |
| 143 | TCP | IMAP | Email sync across devices |
| 443 | TCP | HTTPS | Encrypted web traffic (TLS) |
| 3306 | TCP | MySQL | Database connections |
| 3389 | TCP | RDP | Remote Desktop Protocol (Windows) |

🔐 Security Note: Ports below 1024 are 'well-known' and require root/admin to open. Attackers scan for open ports (using nmap) to find entry points. Always close unused ports and never use Telnet (port 23) on a production system.

CHAPTER 5

Servers, Clients & Network Devices

## Server vs Client

|  | Server | Client |
| --- | --- | --- |
| Role | Provides resources / services | Requests resources / services |
| Power | High-end, always-on hardware | Can be any device |
| Examples | Web server, game server, DNS | Phone, laptop, smart TV |
| IP Address | Usually static (fixed) | Usually dynamic (from DHCP) |

## Network Devices at a Glance

| Device | OSI Layer | What it Does | Key Note |
| --- | --- | --- | --- |
| Hub | Layer 1 | Broadcasts data to ALL devices | Obsolete — replaced by switches |
| Switch | Layer 2 | Sends data only to the correct device | Uses MAC addresses. Fast and efficient |
| Router | Layer 3 | Connects different networks, assigns IPs | Runs DHCP. Your home gateway |
| Firewall | Layer 3/4 | Filters traffic based on rules | First line of defence |
| WAP | Layer 2 | Wireless Access Point — extends to Wi-Fi | Bridges wireless to wired network |

CHAPTER 6

Security Concepts

## The CIA Triad

The three pillars of information security — every security decision maps back to at least one:

| Principle | Goal | How it's Achieved |
| --- | --- | --- |
| C — Confidentiality | Only authorised users can see data | Encryption, access controls, authentication |
| I — Integrity | Data is accurate and unmodified | Hashing (MD5, SHA-256), digital signatures |
| A — Availability | Systems are accessible when needed | Redundancy, backups, DDoS protection |

## Common Network Attacks

| Attack | OSI Layer | Description | Defence |
| --- | --- | --- | --- |
| DDoS | 3 / 4 / 7 | Flood a target with traffic to take it offline | Rate limiting, CDN, firewall rules |
| MITM | 2 / 3 | Intercept communication between two parties | TLS/HTTPS, VPN, certificate pinning |
| Port Scanning | 3 / 4 | Probe open ports to find attack surface (e.g. nmap) | Firewall, close unused ports |
| DNS Spoofing | 7 | Corrupt DNS cache to redirect users to fake sites | DNSSEC, use trusted resolvers |
| ARP Poisoning | 2 | Link attacker's MAC to a legitimate IP on the LAN | Dynamic ARP Inspection (DAI) |
| Phishing | 7 | Trick users into revealing credentials via fake pages | User training, MFA, email filters |

🛡️ Defence Mindset: In Cyber Defence, assume breach. Even with perfect perimeter security, assume an attacker is already inside and design your monitoring and response accordingly. Always audit /var/log, check open ports, and keep software patched.

"Security is not a product, but a process." — Bruce Schneier